1. Constraint-Based Problem Solving: Rural Branch Office Network

1. Problem Identification

The multinational company needs to establish a branch office in a remote rural location where high-speed wired Internet is unavailable. The network must support:

- Cloud-based applications

- Video conferencing

- Secure communication with headquarters

- Continuous business operations

- Limited connectivity budget

- Future expansion

The major constraints are limited bandwidth, unreliable infrastructure, high connectivity cost, security requirements, and scalability.

2. Recommended Internet Connection

Primary connection: 4G/5G Fixed Wireless Access (FWA)
Backup connection: Satellite Internet

A 4G/5G router with an external high-gain antenna can be used as the primary Internet connection if mobile coverage is available.

If cellular connectivity becomes unavailable, a satellite Internet connection can act as a backup.

| Requirement | Recommendation | Reason |
| --- | --- | --- |
| Primary Internet | 4G/5G FWA | Lower cost and good bandwidth |
| Backup Internet | Satellite | Works in remote locations |
| Video conferencing | QoS-enabled connection | Gives priority to real-time traffic |
| Cloud applications | Stable Internet + QoS | Reduces interruptions |
| Headquarters communication | Site-to-site VPN | Provides secure connectivity |

Why not use satellite as the primary connection?

Satellite provides coverage in remote areas, but it can have higher latency and higher recurring costs. This can affect video conferencing and interactive cloud applications. Therefore, it is better used as a backup where affordable cellular coverage exists.

3. Recommended Networking Devices

Network architecture:

4G/5G Internet

[4G/5G Router]

[Firewall/VPN]     [Satellite Backup]

[Managed Switch]

|

PCs   Servers   Wi-Fi AP

|

Employees

Devices required

- 4G/5G Router

  - Provides Internet connectivity.

  - Supports SIM-based connectivity.

  - External antenna can improve signal strength.

- Satellite modem/router

  - Provides backup Internet when the primary connection fails.

- Next-Generation Firewall

  - Controls incoming and outgoing traffic.

  - Provides VPN, intrusion prevention and access control.

  - Blocks unauthorized connections.

- Managed Gigabit Switch

  - Connects computers, printers, access points and other devices.

  - Supports VLANs and QoS.

- Enterprise Wi-Fi Access Points

  - Provides secure wireless connectivity.

  - Supports separate employee and guest networks.

- UPS

  - Keeps networking devices running during short power interruptions.

  - Particularly important in rural locations where power reliability may be lower.

4. Security Mechanisms

Because the branch communicates with headquarters, security should be implemented at multiple levels.

a) Site-to-Site VPN

Create an IPsec site-to-site VPN between the rural branch and headquarters.

Branch Office                    Headquarters

|                                |

[Firewall] ===== Encrypted VPN ===== [Firewall]

This encrypts sensitive company traffic travelling over the public Internet.

b) Firewall

Configure the firewall to:

- Block unauthorized inbound traffic

- Allow only required services

- Detect suspicious traffic

- Apply security policies

c) VLAN Segmentation

Create separate VLANs, for example:

VLAN 10 → Employees

VLAN 20 → Servers

VLAN 30 → Voice/Video

VLAN 40 → Guest Wi-Fi

This prevents a compromised guest device from directly accessing company resources.

d) Secure Wi-Fi

Use WPA3 Enterprise where supported, with strong authentication.

e) Multi-Factor Authentication

Use MFA for cloud applications and remote access to reduce the risk of compromised passwords.

f) Monitoring

Monitor:

- Bandwidth usage

- Link availability

- VPN status

- Security events

- Packet loss and latency

5. Constraint-Based Decision Making

The recommendations can be evaluated against the major constraints.

| Constraint | Solution | Justification |
| --- | --- | --- |
| No wired broadband | 4G/5G FWA | Uses existing cellular infrastructure |
| Limited budget | FWA as primary | Usually cheaper than relying entirely on satellite |
| Limited bandwidth | QoS | Prioritizes video conferencing and business traffic |
| Unreliable connection | Satellite backup | Provides an alternative Internet path |
| Security | Firewall + IPsec VPN | Protects communication with headquarters |
| Rural power issues | UPS | Maintains network during short outages |
| Guest access | VLANs | Separates guest traffic from company network |
| Future growth | Managed switch + scalable APs | Additional users/devices can be added |
| Business continuity | Dual WAN/failover | Automatically switches to backup connection |

6. Bandwidth Management

Since rural connectivity may have limited bandwidth, Quality of Service (QoS) should be configured.

Priority can be assigned as:

Highest Priority

↓

Video Conferencing / VoIP

↓

Business Cloud Applications

↓

VPN Traffic

↓

Normal Web Browsing

↓

Guest/Entertainment Traffic

Lowest Priority

Bandwidth can also be limited for guest users so that they cannot consume the entire Internet connection.

7. Reliability and Failover

The firewall/router should support dual-WAN automatic failover.

Internet

|

--------------------

|                  |

4G/5G              Satellite

PRIMARY             BACKUP

|                  |

------[Firewall]----

|

LAN

If the 4G/5G connection fails, the firewall automatically switches traffic to the satellite connection.

When the primary connection becomes available again, traffic can be switched back.

This provides business continuity without requiring employees to manually change connections.

8. Future Scalability

The proposed design can easily support future expansion.

For example:

- Add additional 4G/5G capacity when available.

- Upgrade to 5G or fiber if wired infrastructure reaches the area.

- Add more Wi-Fi access points.

- Add additional VLANs.

- Increase switch capacity.

- Introduce SD-WAN if multiple branch locations are added.

- Increase VPN bandwidth as the number of employees grows.

Conclusion

The most suitable solution is a 4G/5G Fixed Wireless connection as the primary Internet link, satellite Internet as a backup, and a business-grade firewall/router with automatic failover and IPsec VPN. A managed switch, secure Wi-Fi access points, VLAN segmentation and UPS should complete the branch network.

This solution satisfies the main constraints:

Low cost → 4G/5G primary
Remote location → Wireless/Satellite connectivity
Uninterrupted operation → Automatic failover
Security → Firewall + VPN + VLANs
Limited bandwidth → QoS and bandwidth management
Future growth → Scalable managed network

Therefore, the proposed architecture provides the best balance between bandwidth, reliability, security, cost, and scalability for a rural multinational branch office.

2. Constraint-Based Problem Solving: Campus-Wide Wireless Network

1. Problem Identification

The university plans to provide wireless Internet access for more than 3,000 students across classrooms, laboratories, libraries, and hostels.

The network must:

- Support a large number of simultaneous users

- Provide campus-wide coverage

- Minimize signal interference

- Provide secure access

- Work with a limited number of Access Points (APs)

- Remain within the university's budget

The major constraints are high user density, limited APs, interference, security, bandwidth requirements, and cost.

Therefore, the objective is to design a high-capacity, low-interference, secure, and cost-effective wireless network.

2. Recommended Wireless Network Design

A centrally managed Wi-Fi 6/6E network is recommended.

Network Architecture

Internet

↓

[Core Router]

↓

[Core Switch]

↓

[Wireless Controller]     [Authentication Server / RADIUS]

↓                         ↓

AP1, AP2, AP3, AP4 ... APs

↓

Students / Faculty / Staff

The Wireless LAN Controller (WLC) centrally manages the Access Points, including channel allocation, transmit power, roaming, and network configuration.

The RADIUS server provides centralized authentication for students, faculty, and staff.

3. Access Point Placement

Since the university has a limited number of APs, they should not simply be distributed equally across the campus.

AP placement should be based on user density and expected network traffic.

High-density areas

More AP capacity should be provided in:

- Classrooms

- Laboratories

- Libraries

- Hostels and common areas

- Student gathering areas

Lower-density areas

Fewer APs are required in:

- Corridors

- Administrative areas

- Open spaces with fewer users

Example Placement

CLASSROOM BLOCK

[AP1] → Classroom

[AP2] → Classroom

[AP3] → Lab 1

[AP4] → Lab 2

LIBRARY

[AP5] → High-density coverage

[AP6] → High-density coverage

[AP7] → High-density coverage

APs should preferably be installed at ceiling or elevated positions to provide more uniform coverage.

A wireless site survey should be conducted before deployment to identify dead zones, obstacles, and interference sources.

4. Frequency Band Selection

A combination of 2.4 GHz, 5 GHz, and 6 GHz should be used where Wi-Fi 6E-compatible infrastructure and client devices are available.

| Frequency Band | Recommended Use | Reason |
| --- | --- | --- |
| 2.4 GHz | Basic and legacy devices | Better range and penetration |
| 5 GHz | Main student traffic | Higher speed and more usable channels |
| 6 GHz | High-density modern devices | More available spectrum and less legacy interference |

Recommended Strategy

5 GHz should be the primary band for students because it provides better performance and capacity.

2.4 GHz can be used for devices that require greater range or do not support newer bands.

6 GHz can be deployed selectively in high-density areas such as libraries and modern laboratories.

5. Minimizing Signal Interference

A large campus with many APs can experience interference if the APs are not properly configured.

Channel Planning

The wireless controller should assign channels intelligently.

For the 2.4 GHz band, the commonly used non-overlapping channels are:

Channel 1 → Channel 6 → Channel 11

This reduces overlapping-channel interference.

For 5 GHz and 6 GHz, available non-overlapping channels should be assigned based on the site survey and regulatory availability.

Transmit Power Control

APs should not always operate at maximum power.

If neighboring APs transmit too strongly, they may interfere with one another.

Instead:

[AP1] → Controlled Coverage

[AP2] → Controlled Coverage

[AP3] → Controlled Coverage

The controller can adjust AP power levels so that each AP provides sufficient coverage without unnecessarily overlapping with neighboring APs.

6. Supporting More Than 3,000 Users

Coverage alone is not enough. The network must also be designed according to capacity.

For example, putting one AP in a library may provide coverage but could result in hundreds of students competing for the same AP.

Therefore:

High-density areas should receive additional AP capacity even when their coverage areas overlap.

The overlap is intentional because each AP serves a smaller number of users.

Wi-Fi 6 features such as:

- OFDMA

- MU-MIMO

- BSS Coloring

can improve wireless efficiency in environments with many simultaneous users.

7. Authentication and Security

Security is essential because thousands of users will access the university network.

The recommended authentication method is:

WPA3-Enterprise + 802.1X + RADIUS

Students and staff can authenticate using their individual university credentials.

Student

↓

Wi-Fi Access Point

↓

Wireless Controller

↓

RADIUS Server

↓

Authentication

↓

Secure Internet Access

This is more secure than using a single shared Wi-Fi password because each user has individual authentication credentials.

8. VLAN-Based Network Segmentation

Different types of users should be separated using VLANs.

| VLAN | Users |
| --- | --- |
| VLAN 10 | Students |
| VLAN 20 | Faculty |
| VLAN 30 | Administrative Staff |
| VLAN 40 | Guest Users |
| VLAN 50 | Network Management |

For example, a guest user should not be able to access university servers even though the guest is connected to the same physical wireless infrastructure.

This improves both security and network management.

9. Efficient Use of Limited APs

The university's biggest constraint is the limited number of wireless APs.

Therefore, AP deployment should follow a capacity-based placement strategy.

Instead of:

"One AP should cover one physical area."

Use:

"Each AP should serve a planned number of users based on traffic demand."

For example:

Library       → High AP capacity

Laboratory    → High AP capacity

Classrooms    → Medium/High AP capacity

Hostels       → Capacity based on student density

Corridors     → Minimal AP deployment

Open Areas    → APs only where required

This prevents the university from wasting APs in areas with very few users.

10. Cost Optimization

Because of the budget constraint, the university should not simply deploy the maximum possible number of APs.

However, reducing APs excessively can cause:

- Overloaded APs

- Poor throughput

- High latency

- Frequent disconnections

- Poor student experience

Therefore, the best approach is:

Site Survey → Identify User Density → Prioritize High-Demand Areas → Deploy APs Strategically

This provides:

Fewer unnecessary APs + More capacity where required = Better performance within budget

11. Balancing the Constraints

| Requirement | Proposed Solution | Benefit |
| --- | --- | --- |
| More than 3,000 users | Wi-Fi 6/6E | Supports high user density |
| Limited APs | Capacity-based AP placement | Reduces unnecessary equipment |
| Signal interference | Channel planning | Improves signal quality |
| High bandwidth | 5 GHz/6 GHz priority | Provides better performance |
| Longer range | 2.4 GHz selectively | Improves coverage |
| Secure access | WPA3-Enterprise | Strong authentication |
| Individual login | 802.1X + RADIUS | Secure user authentication |
| User separation | VLANs | Improves security |
| Mobility | Central controller | Supports seamless roaming |
| Future growth | Wi-Fi 6E-ready infrastructure | Easier expansion |

12. Future Scalability

The proposed design should support future increases in students, devices, and applications.

For example:

Current Network

↓

Wi-Fi 6/6E

↓

Additional Students

↓

Additional APs in High-Density Areas

↓

Higher-Capacity Switches

↓

Expanded Campus Network

If the number of users increases, additional APs can be installed specifically in overloaded areas without redesigning the complete network.

The university can also upgrade the backbone and Internet connection as traffic increases.

13. Final Recommendation

The university should deploy a centrally managed Wi-Fi 6/6E wireless network, with Access Points strategically placed according to student density and traffic requirements rather than simply distributing them equally across the campus.

5 GHz should be the primary band for high-speed student traffic, while 2.4 GHz should support longer-range and legacy devices. 6 GHz can be introduced in high-density areas for compatible devices.

To minimize interference, the university should use proper channel planning, transmit-power control, and centralized RF management.

For security, WPA3-Enterprise with 802.1X and RADIUS authentication should be used. VLANs should separate students, faculty, administrative staff, guests, and network management.

The limited AP budget should be handled through capacity-based AP placement, giving more AP capacity to libraries, laboratories, classrooms, and hostels with high user density while avoiding unnecessary APs in low-demand areas.

Overall Solution

High User Density

↓

Wi-Fi 6/6E

↓

Strategic AP Placement

↓

5 GHz / 6 GHz Priority

↓

Channel & Power Management

↓

WPA3-Enterprise + 802.1X + RADIUS

↓

VLAN Segmentation

↓

Secure + Reliable Campus Wi-Fi

Conclusion

This solution provides the best balance between coverage, performance, security, and cost. Instead of simply deploying a large number of APs, the university uses intelligent placement and centralized management to maximize the capacity of each AP. High-density areas receive more wireless capacity, interference is controlled through frequency and power management, and WPA3-Enterprise with RADIUS provides secure individual authentication.

Therefore, the proposed design can efficiently support more than 3,000 students while minimizing interference, maintaining secure access, controlling costs, and allowing future network expansion.

3. Constraint-Based Problem Solving: Secure Network Architecture for a Financial Institution

1. Problem Identification

A financial institution handles highly sensitive customer and banking information, so its internal network must provide strong protection against unauthorized access, malware, data breaches, and internal threats.

At the same time, employees must be able to access:

- Sensitive customer information

- Online banking services

- Internal business applications

- Cloud-based services

The organization also has four major constraints:

- High security – customer and financial data must be protected.

- High performance – security controls should not significantly slow down business applications.

- Compliance – strict security policies and auditing requirements must be satisfied.

- Limited budget – security should be improved without unnecessary infrastructure and operational costs.

Therefore, the objective is to create a defense-in-depth architecture that separates critical resources, authenticates users strongly, detects attacks, and controls traffic efficiently.

2. Recommended Secure Network Architecture

A segmented, firewall-protected and centrally monitored architecture is recommended.

Internet

↓

[Edge Firewall]

↓

[DMZ / Public Zone]

↓

[Web / Banking Application Servers]

↓

[Internal Firewall]

↓

[Core Switch]

↓

┌───────────────────┼───────────────────┐

↓                   ↓                   ↓

[Employee VLAN]     [Customer Data VLAN]   [Management VLAN]

↓                   ↓                   ↓

Employee PCs        Database Servers      IT Administrators

↓                   ↓

[IDS/IPS]       [IDS/IPS]

↓                   ↓

[SIEM / Central Monitoring]

The architecture follows the principle:

"Users should receive only the access they need, and critical systems should never be directly exposed to untrusted networks."

3. Network Segmentation

Network segmentation is one of the most important security controls.

Instead of placing every device and server in a single network, the organization should create separate VLANs/security zones.

Recommended Segments

| Segment | Purpose | Security Level |
| --- | --- | --- |
| Employee VLAN | Employee computers | Medium |
| Customer Data VLAN | Sensitive customer databases | Very High |
| Banking Application VLAN | Banking applications | Very High |
| DMZ | Public-facing services | High |
| Guest VLAN | Guest Internet access | Low |
| Management VLAN | Network/security administration | Very High |

For example:

Employee Network

↓

Employee VLAN

↓

Firewall Policy

↓

Banking Application

↓

Customer Data VLAN

An employee should not directly communicate with the database. Access should pass through authorized application services and firewall policies.

Why segmentation?

If an employee computer becomes infected with malware, segmentation prevents the attacker from automatically reaching critical customer databases.

4. Firewall Deployment

A multi-layer firewall architecture should be used.

Perimeter Firewall

The first firewall protects the organization from external Internet threats.

Internet

↓

Perimeter Firewall

↓

DMZ

Only required public services should be allowed through the perimeter firewall.

Internal Firewall

A second firewall should separate the DMZ and employee networks from critical internal systems.

DMZ

↓

Internal Firewall

↓

Internal Network

↓

Customer Data

This provides defense in depth.

Even if an attacker compromises a public-facing server, the internal firewall prevents unrestricted access to sensitive systems.

5. Authentication Mechanism

The financial institution should use Multi-Factor Authentication (MFA) combined with 802.1X/RADIUS for controlled network access.

Employee authentication

Employee

↓

Username + Password

↓

MFA Verification

↓

RADIUS / Identity Server

↓

Access Granted

MFA can use an authenticator application, hardware security key, or other approved second factor.

Role-Based Access Control

Employees should receive access according to their job role.

For example:

| Role | Access |
| --- | --- |
| Customer Service | Customer records required for their work |
| Finance Staff | Financial applications |
| IT Administrator | Network management |
| Guest | Internet only |

This follows the principle of least privilege.

6. Intrusion Detection and Prevention

An IDS/IPS should monitor network traffic for suspicious activity.

IDS

An Intrusion Detection System identifies:

- Port scanning

- Malware activity

- Suspicious login attempts

- Unusual traffic

- Known attack patterns

IPS

An Intrusion Prevention System can automatically block detected malicious traffic.

Recommended placement:

Internet

↓

Firewall

↓

IDS/IPS

↓

DMZ

↓

Internal Firewall

↓

Internal Network

For critical environments, IDS/IPS monitoring should also be applied to important internal segments.

7. Centralized Security Monitoring

Instead of checking individual devices manually, logs from:

- Firewalls

- Servers

- IDS/IPS

- Authentication systems

- Network devices

should be sent to a centralized SIEM/security monitoring system.

Firewall ───────→

IDS/IPS ────────→

Servers ────────→ [SIEM]

Authentication ─→

Network Devices ─→

↓

Security Alerts

↓

Security Team

This improves incident detection and supports security auditing and compliance requirements.

8. Performance Considerations

Strong security should not create unnecessary network delays.

The following techniques can maintain performance:

a) Hardware-Based Firewall

Use a firewall capable of handling the organization's expected traffic instead of an underpowered device.

b) VLANs

VLANs reduce unnecessary broadcast traffic and improve network organization.

c) Selective Inspection

Not every piece of traffic needs the same level of deep inspection. Critical traffic should receive stronger inspection, while trusted internal traffic can follow optimized policies.

d) Load Distribution

Critical services can use load balancing and redundant servers where required.

Thus:

Security controls + optimized traffic policies = Security without significant performance degradation.

9. Compliance Considerations

Since financial institutions operate under strict regulatory requirements, the architecture should support:

- Strong access control

- Data confidentiality

- Encryption

- User activity logging

- Security event monitoring

- Regular vulnerability assessment

- Audit trails

- Data backup and recovery

Sensitive information should be encrypted both during transmission and while stored.

Security logs should be retained according to the organization's applicable regulatory and compliance requirements.

10. Budget Optimization

The organization has a limited budget, so the solution should avoid unnecessary duplication.

Cost-effective approach

Instead of deploying expensive security appliances everywhere:

- Use VLANs for logical segmentation.

- Use centralized firewall policies.

- Use existing managed switches where possible.

- Centralize logs through one SIEM platform.

- Use open-source or appropriately licensed IDS tools where organizational policy permits.

- Automate security monitoring and alerts.

- Prioritize advanced security controls for critical banking and customer-data systems.

The objective is:

Maximum security improvement with minimum additional infrastructure and operational overhead.

11. Balancing the Constraints

| Constraint | Design Decision | Benefit |
| --- | --- | --- |
| Security | Segmentation + firewall + IDS/IPS | Reduces attack surface |
| Sensitive data | Separate customer-data VLAN | Limits unauthorized access |
| Authentication | MFA + 802.1X/RADIUS | Prevents unauthorized access |
| Performance | Optimized firewall and traffic policies | Minimizes latency |
| Compliance | Centralized logging + auditing | Supports regulatory requirements |
| Budget | VLANs + centralized security management | Reduces infrastructure cost |
| Internal threats | Least privilege + segmentation | Limits lateral movement |
| Future growth | Scalable VLAN and firewall architecture | Easy expansion |

12. Example Attack Scenario

Consider an employee accidentally opens a malicious attachment.

Without segmentation:

Employee PC

↓

Malware

↓

Internal Network

↓

Customer Database

↓

Data Breach

With the proposed architecture:

Employee PC

↓

Malware

↓

Employee VLAN

↓

Firewall / IDS/IPS

↓

Attack Detected / Blocked

↓

Customer Data Protected

This demonstrates why segmentation + firewall + IDS/IPS is more effective than relying on a single security mechanism.

13. Final Recommendation

The financial institution should implement a defense-in-depth network architecture consisting of:

Network Segmentation → Multi-Layer Firewall → MFA + 802.1X/RADIUS → IDS/IPS → Centralized SIEM Monitoring

The most sensitive customer databases and banking applications should be placed in separate protected VLANs/security zones, while public-facing services should be isolated in a DMZ.

Employees should authenticate using MFA and centralized identity management, with role-based access control ensuring that users receive only the permissions necessary for their jobs.

Firewalls should control communication between security zones, while IDS/IPS monitors and blocks suspicious activity. Centralized logging supports incident response, auditing, and compliance.

Overall Solution

External Threats

↓

Perimeter Firewall

↓

DMZ

↓

Internal Firewall

↓

Network Segmentation

↓

MFA + 802.1X Authentication

↓

Least-Privilege Access

↓

IDS/IPS Monitoring

↓

Centralized SIEM

↓

Secure Financial Network

Conclusion

This design provides a strong balance between security, performance, compliance, and cost. Segmentation limits the spread of attacks, firewalls control access between zones, MFA prevents unauthorized users from accessing sensitive resources, and IDS/IPS detects and blocks malicious activity.

At the same time, using VLAN-based segmentation, centralized management, optimized firewall policies, and selective security inspection avoids unnecessary infrastructure and performance overhead.

Therefore, the proposed architecture provides strong protection for customer and financial data while maintaining efficient employee access, satisfying compliance requirements, controlling operational costs, and allowing the network to scale in the future.