Comparative Analysis of Network Topologies

1. Star vs Bus Topology — Cost, Reliability and Performance

| Factor | Star Topology | Bus Topology |
| --- | --- | --- |
| Cost | More expensive because it requires a central switch/hub and individual cables | Less expensive because devices share a single backbone cable |
| Reliability | High; failure of one cable/device affects only that node | Low; failure of the backbone can affect the entire network |
| Performance | Better performance because a switch manages communication and reduces unnecessary collisions | Performance decreases as more devices share the same communication medium |
| Maintenance | Easy to troubleshoot and isolate faults | Difficult to locate backbone faults |
| Scalability | Easy to expand if switch ports are available | Limited because adding devices increases traffic on the shared backbone |

Conclusion: Star topology is more expensive initially, but it provides better reliability, performance, maintenance, and scalability. Bus topology is mainly advantageous when low cost and simplicity are the primary requirements.

2. Mesh vs Star Topology — Fault Tolerance

| Factor | Mesh Topology | Star Topology |
| --- | --- | --- |
| Fault tolerance | Very high because multiple alternative paths exist | Moderate |
| Link failure | Traffic can use an alternate path | Only the connected node is affected |
| Central device failure | No single central device normally controls the entire network | Central switch/hub failure can affect the entire network |
| Reliability | Extremely high | High for individual-link failures |
| Cost | Very expensive due to many connections | Comparatively less expensive |

Conclusion: Mesh topology provides greater fault tolerance than star topology because it provides redundant communication paths. Therefore, mesh is suitable for critical environments such as banking, data centers, and backbone networks, while star is more practical for ordinary LANs.

3. Bus vs Mesh Topology — Installation Complexity

| Factor | Bus Topology | Mesh Topology |
| --- | --- | --- |
| Installation | Simple | Complex |
| Cabling | Requires a common backbone | Requires multiple links between nodes |
| Number of connections | Relatively few | Very large |
| Cost | Low | High |
| Scalability | Limited | Difficult and expensive to scale |
| Maintenance | Backbone failures can be difficult to locate | Large number of connections makes troubleshooting complex |

Conclusion: Bus topology is simpler, cheaper, and faster to install, whereas mesh topology provides superior reliability at the cost of greater installation complexity and expense.

4. Hybrid vs Star Topology — Scalability

| Factor | Hybrid Topology | Star Topology |
| --- | --- | --- |
| Scalability | Very high | High but limited by central device capacity |
| Expansion | New topology segments can be added according to requirements | Additional devices require available switch ports or additional switches |
| Flexibility | Very flexible | Less flexible |
| Large networks | Suitable for campuses and enterprises | Suitable for small and medium LANs |
| Cost of expansion | Depends on the topology added | Usually predictable and manageable |

Conclusion: Hybrid topology provides greater scalability and flexibility because different topologies can be combined according to network requirements. Star topology is easier to manage but may require additional switches when the central switch reaches its capacity.

5. Maintenance Comparison — Star, Mesh, Bus and Hybrid

| Factor | Star | Mesh | Bus | Hybrid |
| --- | --- | --- | --- | --- |
| Maintenance difficulty | Low | High | Moderate/High | Moderate |
| Fault identification | Easy | Complex | Difficult when backbone fails | Depends on topology used |
| Troubleshooting | Simple | Time-consuming | Difficult | Requires skilled administration |
| Effect of cable failure | Usually one node affected | Alternative paths available | Entire segment/network may be affected | Depends on affected segment |
| Best suited for | Office LANs | Critical networks | Small/simple legacy networks | Large enterprise/campus networks |